# Enterprise Industrial IoT Platform Architecture

## 1. Architecture Overview

Industrial Devices / PLC

│

▼

Protocol Gateway Layer (Modbus / OPC-UA / MQTT)

│

▼

Edge Streaming Layer (MQTT or NATS)

│

▼

Processing Layer (Filtering / Workflows)

│

├── Real-time State Cache (Redis)

│

└── Historian Storage (ClickHouse / TimescaleDB)

│

▼

Real-time Delivery Layer (WebSocket Gateway)

│

▼

Visualization Engine (Web dashboards / optional native HMI)

## 2. Gateway Layer

Recommended stack:

- Protocol adapters: Rust / Go / Node.js

- Protocols: Modbus TCP, OPC‑UA, MQTT Sparkplug

Reasoning:

- Handles device polling and reconnection

- Normalizes telemetry

- Buffers device data for resilience

Examples:

- AWS IoT Greengrass

- Azure IoT Edge

- Ignition Edge

## 3. Messaging / Streaming Layer

Recommended stack:

- MQTT broker (device ingestion)

- NATS JetStream (internal event streaming)

Reasoning:

- MQTT is lightweight and ideal for device communication

- NATS provides extremely low-latency internal messaging

- Enables decoupled microservices architecture

Examples:

- Tesla telemetry pipelines (Kafka variant)

- HiveMQ industrial messaging

- Cloudflare streaming systems (NATS)

## 4. Processing Layer

Recommended stack:

- Node.js (Fastify) or Go microservices

Responsibilities:

- Noise filtering

- Delta detection

- Unit normalization

- Alarm detection

- Workflow triggers

Examples:

- Azure IoT event processors

- ThingsBoard rule engine

- Uber streaming pipelines

## 5. Real-Time State Cache

Recommended stack:

- Redis

Reasoning:

- In-memory performance

- Pub/Sub support

- Real-time dashboard fan-out

Examples:

- GitHub caching infrastructure

- Grafana state cache

- Cloudflare Redis clusters

## 6. Historian Layer

Recommended stack:

- ClickHouse or TimescaleDB

Reasoning:

- Handles high ingestion rates

- Efficient analytical queries

- Columnar compression

Examples:

- Cloudflare analytics (ClickHouse)

- Uber metrics platform (ClickHouse)

- IoT analytics platforms (TimescaleDB)

## 7. Real-Time Delivery Layer

Recommended stack:

- WebSocket Gateway

Architecture:

Redis Pub/Sub → WebSocket Gateway → Dashboard Clients

Reasoning:

- Prevents API server overload

- Enables efficient fan-out to many clients

Examples:

- Grafana Live dashboards

- Trading dashboards

- Industrial monitoring systems

## 8. Visualization Engine

Recommended stack:

- Next.js

- React

- TypeScript

Charts:

- uPlot

- Apache ECharts

Reasoning:

- Rich UI capabilities

- Efficient rendering

- Strong ecosystem

Examples:

- Grafana dashboards

- Datadog monitoring

- ThingsBoard visualization

## 9. Optional High-End Visualization

Optional stack:

- Three.js

Use cases:

- 3D plant layouts

- Digital twins

- Robotics visualization

## 10. Minimum Viable Infrastructure

Single-server deployment:

Services:

- Gateway

- Processing engine

- NATS

- Redis

- ClickHouse

- API

- WebSocket Gateway

- Frontend

Minimum specifications:

- CPU: 8 cores

- RAM: 32 GB

- Storage: NVMe SSD

Supports thousands of sensors.

## 11. Comparison with Tatsoft FrameworX

Tatsoft Architecture:

Device Drivers → Tag Engine → Historian → HMI

Characteristics:

- Centralized tag database

- Traditional SCADA model

Your Architecture:

Device → Streaming pipeline → Processing → Cache → Visualization

Advantages:

- Horizontal scalability

- Cloud-native deployment

- Microservices architecture

- Native streaming analytics

## 12. Comparison with Unreal Engine Visualization

Unreal Architecture:

Industrial data → Middleware → Unreal visualization

Advantages:

- Advanced 3D graphics

- Digital twin environments

Disadvantages:

- Heavy infrastructure

- GPU requirements

- Complex deployment

Your Platform Advantages:

- Lightweight

- Easy deployment

- Optimized for telemetry dashboards