Since you must treat the upstream systems as a black box and only design your application layer, the architecture you start at the protocol gateway boundary (OPC-UA / Modbus).

The goal is to clearly show:

- your system ingests telemetry

- processes it in real time

- stores historical data

- streams live updates to dashboards

- scales for high-frequency signals

Below is a clean architecture description you can feed directly to Claude.

# Architecture Scope Definition (Important)

Your system starts at the industrial protocol interface.

Everything before that is treated as an external system.

External System Boundary

├─ OPC-UA Gateway

└─ Modbus TCP Gateway

Your platform connects to these interfaces to receive telemetry data.

# System Architecture (Application Layer)

Industrial Gateway Layer

(OPC-UA / Modbus TCP)

│

▼

Protocol Adapter Layer

│

▼

Telemetry Ingestion Service

│

▼

Internal Streaming Bus

│

├──────────────► Real-Time Processing Engine

│                    │

│                    ▼

│              Redis Real-Time Cache

│                    │

│                    ▼

│              WebSocket Gateway

│                    │

│                    ▼

│                 Dashboard

│

▼

Storage Worker

│

▼

MongoDB Time-Series Database

# 1. Protocol Adapter Layer

Purpose: connect to industrial gateways.

Supported protocols:

- OPC-UA (preferred)

- Modbus TCP (fallback)

Responsibilities:

- establish connections to gateways

- subscribe to OPC-UA nodes

- poll Modbus registers when required

- normalize incoming telemetry data

- forward data to ingestion service

Example normalized message:

{

"sensorId": "turbine.speed",

"timestamp": 1719982012123,

"value": 7420.3,

"quality": "GOOD"

}

The protocol layer abstracts industrial protocols from the rest of the platform.

# 2. Telemetry Ingestion Service

Responsibilities:

- receive normalized telemetry events

- validate payload structure

- enrich metadata (deviceId, sensor group, units)

- publish events to the internal stream bus

Design principles:

- stateless

- horizontally scalable

- minimal processing

This layer ensures ingestion remains lightweight and fast.

# 3. Internal Streaming Bus

Purpose: decouple ingestion, processing, storage, and visualization.

Possible technologies:

- NATS JetStream

- Apache Kafka

Streams:

sensor.raw

sensor.processed

sensor.alerts

Benefits:

- buffering

- parallel processing

- replay capability

- backpressure handling

# 4. Real-Time Processing Engine

Consumes telemetry from the stream bus.

Responsibilities:

- noise filtering

- small-change detection

- rule evaluation

- window aggregation

- derived metrics

Example processing:

delta = abs(current - previous)

if delta > threshold

emit update

Output stream:

sensor.processed

# 5. Redis Real-Time Cache

Purpose: serve the latest values quickly for dashboards.

Structure example:

sensor:latest:{sensorId}

Example value:

{

"value": 7420.3,

"timestamp": 1719982012123

}

Advantages:

- sub-millisecond reads

- reduces load on MongoDB

- supports pub/sub for live updates

# 6. WebSocket Gateway

Dedicated service responsible for pushing real-time updates to dashboards.

Responsibilities:

- subscribe to Redis updates

- manage client subscriptions

- broadcast sensor updates to relevant clients

Clients can subscribe to:

sensorId

deviceId

sensorGroup

This prevents unnecessary data transmission.

# 7. MongoDB Time-Series Storage

Used for historical telemetry storage and analytics.

Collection example:

db.createCollection("sensor_data", {

timeseries: {

timeField: "timestamp",

metaField: "sensorId",

granularity: "seconds"

}

})

Example document:

{

"sensorId": "turbine.speed",

"timestamp": ISODate("2026-03-06T12:10:10Z"),

"value": 7420.3

}

Storage workers batch writes for efficiency.

Batch configuration example:

batch size: 1000

flush interval: 200 ms

# 8. Dashboard Data Strategy

Dashboards use two data paths.

Real-time monitoring:

WebSocket → Redis → processed telemetry

Historical queries:

REST API → MongoDB time-series

Charts combine:

- live stream data

- historical database queries

# 9. Scaling Strategy

The architecture supports horizontal scaling.

Services that can scale independently:

- protocol adapters

- ingestion services

- stream processors

- storage workers

- WebSocket gateways

Partitioning strategy:

partition key = sensorId hash

This distributes load evenly.

# 10. Performance Target

Designed to support:

10,000+ sensors

25 Hz sampling rate

≈250,000 updates per second

Latency targets:

| Component | Target |
| --- | --- |
| Protocol adapter | <10 ms |
| Ingestion | <5 ms |
| Stream propagation | <10 ms |
| WebSocket delivery | <20 ms |

Total expected dashboard latency:

≈30–50 ms

# 11. System Design Principles

The architecture follows these principles:

- protocol abstraction from industrial systems

- stream-first data pipeline

- separation of ingestion, processing, and storage

- real-time cache for dashboard performance

- horizontal scalability

- resilience through buffering and decoupling